Importing & Processing a Leica TS02/06/08 Traverse using FlexOffice

Version 1.0

Not only does TraversePRO support different traversing methods in the field you can additionally adjust traverse observations using the Compass/Transit methods to distribute coordinate misclosures, 2D-Helmert-Transformation for traverses using just two control points and the options Equal/By Distance/No are available to distribute any height misclosure. Note any angular misclosures are always distributed equally. For further details on how to process your data onboard, please refer to the TS02/06/08 User Manual, subchapter TraversePRO (optional) under PROGRAMS.

To view your TS02/06/08 adjusted traverse results or recalculate your TS02/06/08 traverse using a range of different processing parameters we now provide FlexOffice Standard and FlexOffice Advanced software. The following steps are an overview on how to import and view LandXML traverse measurement and fix point data in FlexOffice.

# FlexOffice Standard

- FlexOffice Standard allows you to view the adjusted point information calculated by the onboard application TraversePro in the View/Edit module of FlexOffice. FlexOffice Standard does not include the TPS Processing module. We therefore recommend importing the Fix point XML file of the traverse job to view the adjusted traverse data.

- All points with the prefix C are the adjusted points.

# FlexOffice Advanced

- When using TPS Processing module in FlexOffice Advanced we recommend you export the traverse job using the XML format from the instrument.

- In FlexOffice Advanced import the relevant measured XML file only. The fix point file is not needed.

- FlexOffice Advanced automatically reads through the measured XML file for all traverse processing parameters as previously set in the field. The traverse processing function inside FlexOffice will then automatically calculate and adjust the traverse.

- To see your results got to TPS Processing > Traverses > right click on your traverse > select Traverse Report

- If no results appear in the traverse report then it is most likely FlexOffice does not support processing that type of traverse. It would still then be possible to simply import your the fixpoint XML file and view the adjusted points in View/Edit.

- Note it is also possible to create a traverse in FlexOffice Advanced where the TraversePro application was not used on the instrument by connecting a series of setups with the help of the FlexOffice Traverse Wizard.

- For more information on traversing, supported traverse techniques and TPS Processing in Flex Office please refer to the FlexOffice Help Menu under Projects.